|  | ## Kendriya Vihar-II Apartments Owners’ Association # Community Center, Kendriya Vihar, Plot No.3, Sector-82, NOIDA-210304 (U.P.) # Ph-0120-2463700 |
| --- | --- |

No.AOA/Road Repair/2011-12/				          Dated:   12-12-2011

INVITATION OF TENDER

Sealed and subscribed quotations are invited from eligible contractors by the Administrator, Kendriya Vihar-II Apartment Owners’ Association, (KV-II AOA), Sector 82, NOIDA for repair of roads inside the Kendriya Vihar-II complex as per schedule attached at Annexure-1 and terms and conditions mentioned below.

Last date of sale of tender form in KV-II AOA Office at Community Centre 1, Kendriya Vihar-II, Sector 82 NOIDA - 18/12/2011 (upto 7.00 p.m.)

Last date of submission of completed tender form with EMD and required papers in support of eligibility of the contractor – 19/12/2011 (upto 7.00 p.m.)

Terms and Conditions

- The tender form can be purchased from KV-II AOA Office for Rs. 1000/- which is non-refundable. The form can also be downloaded from the website of KV-II AOA, and in that case, a separate demand draft of Rs. 1000/- is required to be deposited along with the tender form.

- The contractor duly registered with any Government Department/ Agency and having experience for carrying single road work costing more than Rs. 50.00 lakh shall be eligible to quote the rates (both in words and figures) for the work and quantity as per attached schedule.

- The contractor has to deposit, along with the tender, a demand draft of Rs. 75,000/-in favour of Kendriya Vihar-II Apartment Owners’ Association (KV-II AOA), Sector 82, NOIDA as Earnest Money Deposit (EMD). The EMD of all unsuccessful contractors shall be returned after award of the work to successful bidder. EMD of successful bidder shall be adjusted against security deposit in lieu of performance.

- In case the lowest bidder refuses to execute the work as per quoted rates, EMD will be forfeited.

- The successful contractor has to deposit 5% of the bid amount as performance guarantee before start of the work, which shall be returned after satisfactory completion of the work.

- 5% of the bid amount shall be deducted from the bill of due payment to the contractor and the same shall be refunded to the contractor after 6 months of completion of the work.

- The work shall be carried out as per CPWD specification with upto date correction slip.

- Sample of pre mix carpet including compaction shall be got approved before start of the work

- Net wt of all bituminous mix will be taken for each tipper in presence of representative of KV-II AOA before execution of the work.

- The theoretical will be done on the basis of density of bituminous mix as 2.30 gm/C.C.

- Time period allowed for execution of the work will be 30 days from award of the work.

- The running payment shall be made as per minimum work done of Rs.25.00 lakh gross and as per satisfaction of the Administrator, KV-II AOA.

- Recoveries @ 4% for vat, @ 2% for income tax and for other applicable taxes/levies shall be made from the bill of payment to the contractor.

- The Administrator, KV-II AOA, Sector 82, NOIDA reserves the right to reject the tender at any point of time without assigning any reason for the same.

Encl: - As above

(Navneet Soni)

Administrator, (KV-II AOA)

Annexure-1

| SCHEDULE OF QUANTITIES |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SN | Items | Qty | Unit | Rate |  | Amount |
|  |  |  |  | In Fig | In Words |  |
| 1 | Providing and applying tack coat using hot straight run bitumen of grade 80/100 including heating the bitumen, spraying the bitumen with mechanically operated spray unit fitted on bitumen boiler, cleaning and preparing the existing road surfaces as per specifications on bituminous surfaces @0.50 Kg/Sqm | 34000 | Sqm |  |  |  |
| 2 | 2.5cm premix carpet (supplied from hot mix plant) surfacing with 2.25cum and 1.12 cum of stone chipping of 13.2 mm and 11.2 mm size respectively per 100 sqm and 52 kg and 56 kg of hot bitumen per cum of stone chippings of 13.2 mm and 11.2 mm size respectively including a tack coat with hot straight run bitumen including consolidation with road roller of 6 to 9 ton capacity etc. complete (tack coat to be paid separately) with paving Asphalt 80/100 heated and then mixed with solvent @ 70 g per kg of Asphalt. | 34000 | Sqm |  |  |  |
| 3 | Providing and laying Bituminous Macadam on prepared surface with specified grade crushed stone aggregate for profile corrective base/binder course including loading of aggregate with FE loader, hot mixing of stone aggregates and bitumen in hot mix plant, transporting the mixed material by tipper to paver and laying the mixed material with paver finisher fitted with electronic sensing device to the required level and grade and rolling with road rollers, as per MORTH specification to achieve the desired density and compaction but excluding the cost of primer/tack coat. | 10 | Cum |  |  |  |
| 4 | Providing and laying in position ready mixed concrete manufactured in fully automatic batching plant and transported to site of work in transit mixer for a lead up to 10kms having continuous agitated mixer, manufactured as per mix design of specified grade for reinforced cement concrete work including pumping of R.M.C. from transit mixer to site of laying, excluding the cost of centering, shuttering finishing and reinforcement including cost of admixtures in recommended proportions as per IS:9103 to accelerate/ retard setting of concrete, improve workability without impairing strength and durability as per direction of the Engineer-in-charge. M-25 grade Reinforced cement concrete by using 380kg of cement per cum of concrete. All works up to plinth level. | 20 | Cum |  |  |  |
|  | Signature of Election Officer/Administrator, KV-II AOA |  |  | Signature of Contractor |  |  |